SIMATS ENGINEERING

ASSESSMENT TOOL 1- INDUSTRY PROBLEM SOLVING TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL1- INDUSTRY PROBLEM SOLVING TASK |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | V Jeevanantham | Reg.No.: | 192421234 |
| Department: |  | Date: |  |

ANNEXURE A

INDUSTRY PROBLEM SOLVING TASK

- Smartphone Performance Optimization                                 
 A mobile manufacturer observes lag while switching between apps. Analyze the role of L1, L2, L3 cache and DRAM and propose a hierarchical memory redesign to reduce latency and improve throughput.

- Cloud Server Memory Bottleneck
 A cloud service provider experiences high latency in database queries. Compare cache hierarchy vs virtual memory paging and recommend improvements using appropriate memory technologies.

- Autonomous Vehicle Data Processing
 An autonomous vehicle must process sensor data in real time. Evaluate SRAM, DRAM, and MRAM and design a memory hierarchy that maximizes bandwidth and minimizes latency.

- AI Inference Edge Device
 An edge AI device needs fast model loading with low power consumption. Compare NAND Flash, DRAM, and RRAM and recommend an optimized hierarchical memory structure.

- High-Performance Gaming Console
 A gaming console suffers frame drops during large scene loading. Analyze L3 cache vs DRAM throughput and propose memory hierarchy enhancements.

Code of Conduct:

I V Jeevanantham / 192421234 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Industry Problem | 20% | Comprehensive understanding of real-world problem and constraints | Good understanding with minor gaps in requirements | Basic understanding; some requirements unclear | Poor understanding; misinterprets problem context |
| Application of Engineering Concepts | 25% | Applies advanced and relevant concepts effectively to solve the problem | Applies appropriate concepts with minor errors | Limited application of concepts | Incorrect or no application of concepts |
| Solution Design | 25% | Innovative, practical, and feasible solution aligned with industry standards | Logical and feasible solution with minor gaps | Basic solution with limited feasibility | Unclear or impractical solution |
| Use of Tools | 15% | Effective use of modern tools, technologies, and real data | Appropriate use of tools with correct implementation | Limited or inconsistent use of tools | Minimal or incorrect use of tools |
| Analysis | 10% | Thorough analysis with validated results and performance evaluation | Good analysis with reasonable validation | Basic analysis with limited validation | Poor or no analysis |

1. Understanding of the Problem: App-switching lag stems from repeated cache misses and slow DRAM fetches whenever the OS reloads app context, since the working set of the newly foregrounded app is rarely still resident in the fast cache tiers.

Application of Engineering Concepts: Applying memory hierarchy principles, the fix targets the tiers by access frequency: keep hot, per-core data in L1 (fastest, smallest), near-term reusable data in L2, and data shared across recently used apps in a larger shared L3, backed by DRAM for bulk storage.

Solution Design: Redesign proposal: L1 32-64 KB (~1-2 ns) per core, L2 256 KB-1 MB (~4-6 ns) per core, and a shared L3 of 4-8 MB (~10-15 ns) sized to hold multiple apps' hot data simultaneously; pair with an app-aware prefetcher that pre-warms cache lines for apps likely to be switched to next, plus low-power LPDDR5 DRAM as the backing store.

Use of Tools/Technologies: Use cache profiling/simulation tools (e.g., gem5, Cachegrind) to model hit/miss rates for the redesigned hierarchy, and Android's ART/JIT profiling data to identify which app pages should be prioritized for cache residency.

Analysis: Simulated hit-rate improvement from added L3 capacity and prefetching is expected to cut average memory access time significantly, directly reducing the perceived lag during app switches while keeping the added power draw of LPDDR5 minimal compared to a full DRAM bandwidth increase.

2. Understanding of the Problem: High query latency in the cloud database points to the working set exceeding cache capacity, forcing frequent page faults and paging traffic between DRAM and slower secondary storage.

Application of Engineering Concepts: Cache hierarchy (SRAM-based L1-L3) gives nanosecond latency but limited capacity, ideal for hot indexes and frequently accessed rows; virtual memory paging extends addressable space using DRAM plus disk but adds page-fault and TLB-miss latency, so it is only appropriate for the larger, colder working set.

Solution Design: Expand L3 cache and enlarge the database buffer pool to keep more of the working set in DRAM; replace the page/swap backing store with NVMe SSDs built on NAND Flash instead of traditional disk to cut page-fault penalty; apply cache partitioning per query thread to avoid cross-query eviction.

Use of Tools/Technologies: Use database buffer-pool monitoring (e.g., MySQL/PostgreSQL cache-hit statistics), OS-level page-fault counters (perf, vmstat), and DDR5/HBM DRAM modules for the working-set tier.

Analysis: Increasing DRAM buffer-pool size and moving swap to NVMe/NAND is expected to reduce page-fault frequency and its associated latency spikes, while per-thread cache partitioning improves L3 hit rates under concurrent query load, together lowering average query response time.

3.Understanding of the Problem: Real-time sensor processing requires consistently low latency and high bandwidth, plus retention of critical safety state even through momentary power fluctuations — a combination no single memory technology satisfies alone.

Application of Engineering Concepts: SRAM gives sub-nanosecond access but is costly and low-density, suiting only immediate sensor buffers in L1/L2; DRAM offers high bandwidth and capacity for frame/sensor-fusion data but is volatile and power-hungry; MRAM combines near-SRAM speed with non-volatility, suited to safety-critical state that must survive power loss.

Solution Design: Proposed hierarchy: SRAM-based L1/L2 cache tightly coupled to the sensor-fusion processor for immediate buffering, DRAM as the main working memory for real-time frame and LIDAR/radar data, and MRAM for non-volatile storage of safety-critical control states so no data is lost during transient power events.

Use of Tools/Technologies: Use real-time OS scheduling with memory-mapped sensor buffers, hardware-in-the-loop simulation to validate latency budgets, and MRAM modules rated for automotive-grade endurance and temperature range.

Analysis: The layered design keeps the sensor-fusion latency budget within the SRAM/DRAM tiers while MRAM's non-volatility eliminates the risk of losing safety state during power glitches, meeting both the bandwidth and reliability requirements of autonomous operation.

4.Understanding of the Problem: Edge AI inference needs fast model loading and low power draw, but current NAND Flash-to-DRAM loading paths introduce latency and DRAM keeps drawing power to retain the model, neither of which fits a battery-constrained edge device.

Application of Engineering Concepts: NAND Flash is dense and non-volatile but has higher read latency and limited write endurance, suiting only cold storage of trained weights; DRAM is fast but volatile and power-hungry during active inference; RRAM is non-volatile, faster than NAND Flash, and low-power, making it well suited to in-memory computation of weights.

Solution Design: Proposed structure: RRAM as the primary non-volatile, low-power tier holding and directly computing with model weights (enabling near-instant loading without a full DRAM copy step), a small DRAM buffer for intermediate activations during inference, and NAND Flash reserved for archival storage of multiple alternate models.

Use of Tools/Technologies: Use in-memory-computing RRAM crossbar arrays for weight storage/multiply-accumulate operations, TensorFlow Lite/ONNX Runtime for quantized edge-model deployment, and power profiling tools to validate energy savings versus a DRAM-only baseline.

Analysis: Shifting weight storage and compute to RRAM removes the DRAM-loading step from the critical path, cutting model-load latency, while RRAM's lower standby power compared with DRAM reduces overall energy consumption during idle and inference periods.

5. Understanding of the Problem: Frame drops during large scene loads occur because DRAM bandwidth cannot keep pace with the GPU's texture/geometry streaming demands, while the fast L3 cache is too small to hold the bulk of scene assets.

Application of Engineering Concepts: L3 cache (low latency, high bandwidth) is best suited to frequently reused small data such as shader instructions and render state; large scene assets like textures and geometry far exceed its capacity and must stream through high-bandwidth DRAM (GDDR6/HBM2), so throughput during load spikes is bounded by DRAM bandwidth, not cache speed.

Solution Design: Enhancements: increase L3 cache size with smarter victim caching to retain hot render-state data, adopt GDDR6/HBM2 DRAM for higher sustained bandwidth during scene transitions, implement asynchronous asset prefetching to overlap loading with rendering, and use compressed texture formats to reduce the DRAM bandwidth needed per frame.

Use of Tools/Technologies: Use GPU profiling tools (e.g., Nsight Graphics, RenderDoc) to identify bandwidth bottlenecks during scene loads, and asset-streaming middleware to pipeline texture prefetch with the render loop.

Analysis: Combining a larger, smarter L3 with higher-bandwidth DRAM and asynchronous prefetching is expected to smooth out the bandwidth spike during scene loads, reducing stalls and frame drops while compressed textures further ease pressure on DRAM bandwidth.